Module 10: Wireless Networks

# Lesson 1: Wireless Networks and Protocols

# Overview.

The importance of wireless networks has never been greater than it is now. The road to wireless networks has been a long, interesting one. Wireless networks started as a humble wireless telegraph in 1896, and grew into telephones, satellites, computers, the ARPANET, ALOHANET, and, eventually, Wi-Fi (Admin, 2017) (Exigent Networks, 2018). Unsurprisingly, it does not seem to be leaving any time soon. In fact, according to market research conducted by PR Newswire the revenue in the global wireless market is expected to expand from $1,431.3 billion in 2019 to $5,519.4 billion by 2024 (Research and Markets, 2020). With the recent pandemic, the position of wireless networks and wireless internet access as an important staple in homes and the workplace has never been more proven and solidified.  As more and more wireless devices become available, wireless networks are becoming more prevalent and, in some cases, even preferred. Imagine, for example, if all the computing devices around had to be connected to share data. That means anything that communicates to other devices. How inconvenient would it be if a phone had to be connected to a smart watch, headphones, laptop, and another person’s phone to send data to all those devices? One would have to constantly move cables and reconfigure devices just to send that meme to your friends. Yet they still wouldn’t have Internet access until you plugged into wired IT infrastructure! With this knowledge, it is vital that our future workforce understands wireless networks, how they work, and why they are needed.

# Objectives.

In this lesson, students learn how to:

- Define a wireless network and its benefits

- Compare and contrast wireless and wired networks to identify advantages and disadvantages of each and determine when each should be used

- Identify types of wireless networks, their benefits/differences and when they are used

- Choose a model that is most appropriate for a given, straightforward scenario

- Match common wireless protocols/standards with their uses

# 3. Discussion Topics.

# Predictions of the Future of Wireless Networking.

- Based on the history of wireless networks and computing, discuss what developments in wireless networking may come about in the future and why would they be needed/what event or technology will initiate these developments.

# Advantages and Disadvantages of Wireless Networks

- Advantages of Wireless Networks

- Introduce the terms scalability and BYOD.

- Students should be able to identify the benefits of wireless networks, such as its ability to increase mobility and access/availability to networks, typically reducing costs to upgrade or expand the network.

- Note also that the benefits of wireless networks extend to the general welfare of society. Public locations with wireless networks provide Internet connectivity to community members which may otherwise be unable to access the Internet. In doing so, they open the door for these citizens to expansive opportunities in employment, education, and beyond; offering these hotspots can also attract more patrons to their business.

- Disadvantages of Wireless Networks

- Point out that, while the benefits of wireless networks are vast, the disadvantages are severe. In addition to reliability and interference issues, wireless networks face critical security vulnerabilities, and their wireless nature creates more opportunity for threat actors to access said networks. Some of the population are fearful, even cripplingly so, of the inherent insecurity of wireless networks, especially those in public locations. Couple this with a fear of technology some have, and wireless networks become a frightening scene for those who could easily fall victim to an attack.

# Network Types and Encryptions

- Discuss why or when a less-secure encryption method may want to be used in specific scenarios, like catching or luring attackers.

- Based on the information provided on network types, discuss what wireless network types may be needed or beneficial and what area they would cover.

# Protocols in Daily Application

- Based on the student’s experience, discuss what protocols may be used when connecting to the Internet or other devices in various locations.

- Using the student’s understanding on interference, waves, and frequency ranges, discuss why having more channels could be better and in what scenarios it could be worse.

# 4. Lecture Discussion Questions.

- Where would you expect to see a wireless network?

Answers will vary. Can include home, office, library, school, café, public places, hard-to-access or cable locations, etc.

- What are some uses or features that may be better suited for a wireless network than a wired network? Why?

Answers will vary. Can include file transfer because you do not have to depend on the parties being close/local or on cable, printing because you do not have to plug in a device each time you want to print from it, security systems because it may be difficult or impossible to run cable to the cameras or alarms, systems where there are many connections because the number of connections is not limited to the number of ports available, etc.

- How do you use wireless networks in daily life?

Answers will vary. Can include smart TV, laptop/computer/other device, phone, Bluetooth speakers/headphone, car, thermostat/smart home technology, etc.

- What is some of the hardware used in wireless networks?

Answer: Wireless-capable router, wireless adapters/NIC, switches, access points, wireless extenders, firewalls, etc.

- Name the steps in how a wireless network works.

Answer: Send data via radio waves to wireless router, radio waves are converted to machine-readable binary, router sends the data out of its wired connection, etc.

- Sort the network types in order of size and discuss what they are used for.

Answer: WPAN (used for small ranges of about 30 feet), WLAN (used for single buildings or a small group of buildings), WMAN (used within a city or region, spans 30 miles), and WWAN (spans beyond cities and can be worldwide)

- Which wireless security method is the safest and why is wireless security so important?

Answer: WPA2; so important because of the inherent security risks with wireless (mentioned in the fourth discussion topic above)

# 5. Additional Suggested Projects/Activities.

- Create and demonstrate the set-up, configuration, and testing of a home router, preferably wireless, and/or wireless network. Once set-up and configuration is complete, show students that the network now shows up on your phone or device, and connect to the network.

- Use Packet Tracer to observe the path of a packet in a wireless network.

- Use Wireshark, if possible, to observe packets in a wireless network. For advanced demonstrations, consider showing how WEP can easily be decrypted in personal mode (wiki.wireshark.org/HowToDecrypt802.11; www.shortmsgserv.co.uk/cracking-wep-encryption/)

- Create a diagram of a wireless network at each level (WPAN, WLAN, WMAN, WWAN).

# 6. Additional Resources.

In addition to the lesson and resources provided above, additional resources may be required or desired. Below are just a few additional resources, though there are a plethora of options available of various complexities.

How the Internet Works in 5 Minutes: www.youtube.com/watch?v=7_LPdttKXPc

WEP and Why You Shouldn’t Use It: www.youtube.com/watch?v=ab6GYvKxfV0; www.coursera.org/lecture/it-security/wep-encryption-and-why-you-shouldn-t-use-it-rVi6O

WPA and WPA2: www.youtube.com/watch?v=hLQ5rYNUwNg; www.youtube.com/watch?v=DspgyuedICM

How Wireless Networking/Wi-Fi Works: www.youtube.com/watch?v=dNsV6Biw4tc; www.youtube.com/watch?v=xKewKMupA58

History of Wireless: www.youtube.com/watch?v=osljGgqYL5w; www.youtube.com/watch?v=X5jPoQzEh-M; www.youtube.com/watch?v=PU6JO8KPRs0

Orthogonal Frequency-Division Multiplexing: en.wikipedia.org/wiki/Orthogonal_frequency-division_multiplexing

Multi-Input Multi-Output: en.wikipedia.org/wiki/MIMO

Direct-Sequence Spread Spectrum: en.wikipedia.org/wiki/Direct-sequence_spread_spectrum

# 7. Key Terms.

Algorithm: a process or set of rules to be followed in calculations or other problem-solving operations, especially by a computer.

Authentication: the process or action of verifying the identity of a user or process.

Automated: the use of largely automatic equipment in a system.

Bit: a unit of information expressed as either a 0 or 1 in binary notation.

Bring Your Own Device (BYOD): the practice of allowing the employees of an organization to use their own computers, smartphones, or other devices for work purposes.

Cellular Network: a radio network distributed over land through cells where each cell includes a fixed location transceiver known as a base station.

Commercial: making or intended to make a profit.

Encryption: the process of converting information or data into a code, especially to prevent unauthorized access.

Hotspot: a public place with an available wireless signal for internet access.

Network Interface Card (NIC): a component that provides networking capabilities for a computer.

Network: a number of interconnected computers, machines, or operations.

Protocol: a set of rules governing the exchange or transmission of data between devices.

Telegraph: a system for transmitting messages from a distance, especially one creating signals by making and breaking an electrical connection.

Transistor: a semiconductor device with three connections, capable of amplification in addition to rectification.

# 8. References.

AccessAgility Blog Team. (2017, December 15). Basic hardware required for a wireless network. Retrieved from AccessAgility: https://www.accessagility.com/blog/basic-hardware-required-for-a-wireless-network

ActionTec. (n.d.). What is a wireless network? Retrieved from ActionTec: https://www.actiontec.com/wifihelp/wifi-networking/what-is-a-wireless-network/

Admin. (2017, May 18). The History of WiFi: 1971 to Today. Retrieved from Cablefree: https://www.cablefree.net/wireless-technology/history-of-wifi-technology/

Arocho, C. (2019, November 12). WaveLAN: the tech that brought Wi-Fi to the world. Retrieved from Bits & Chips: https://bits-chips.nl/artikel/wavelan-the-tech-that-brought-wi-fi-to-the-world/

asawin. (n.d.). Wireless World. Pxhere. Pxhere.

Borth, D. (2017, October 11). Mobile telephone. Retrieved from Encyclopaedia Britannica : https://www.britannica.com/technology/mobile-telephone#ref1079045

Bradley, T. (2020, February 2). Wireless Networking Protocols Explained. Retrieved from Lifewire: https://www.lifewire.com/wireless-networking-protocols-explained-2486947

BT & Cyber Security Challenge UK. (2019, August). Wired and Wireless Networks. Retrieved from Cyber Security Challenge UK: https://www.cybersecuritychallenge.org.uk/app/uploads/2019/08/Lesson-Plan-Wired-and-Wireless-Networks.pdf

CableFree Admin. (2017, May 18). The history of WiFi: 1971 to today. Retrieved from CableFree: https://www.cablefree.net/wireless-technology/history-of-wifi-technology/

CaptainGummyBearz. (2020, November 11). W-Fi. Retrieved from Wikipedia: https://en.wikipedia.org/wiki/Wi-Fi

Chan, A. S. (2018, July 24). A brief history of 1G mobile communication technology. Retrieved from Xoxzo: https://blog.xoxzo.com/en/2018/07/24/history-of-1g/

Cisco. (2020, March 9). Cisco annual internet report (2018–2023) white paper. Retrieved from Cisco: https://www.cisco.com/c/en/us/solutions/collateral/executive-perspectives/annual-internet-report/white-paper-c11-741490.html

Cisco. (n.d.). What is a wireless network? Retrieved from Cisco: https://www.cisco.com/c/en/us/solutions/small-business/resource-center/networking/wireless-network.html

College of Engineering University of Hawaii at Manoa. (n.d.). AlohaNet. Retrieved from College of Engineering University of Hawaii at Manoa: https://www.eng.hawaii.edu/about/history/alohanet/

Commotion Wireless. (n.d.). Types of Wireless Networks. Retrieved from Commotion Wireless: https://commotionwireless.net/docs/cck/networking/types-of-wireless-networks/

(n.d.).Communication Tower. Pxfuel. Pxfuel.

ComputerNetworkingNotes. (2018, August 6). Types of wireless network explained with standards. Retrieved from Computer Networking Notes: https://www.computernetworkingnotes.com/ccna-study-guide/types-of-wireless-network-explained-with-standards.html

Ctia. (n.d.). The wireless industry: industry data. Retrieved from Ctia: https://www.ctia.org/the-wireless-industry/infographics-library/

Customer 1st Communications. (2017). The pros and cons of wired and wireless networks. Retrieved from Customer 1st Communications: https://www.c1c.net/blog/wired-or-wireless-networks-business/

CyberXRef. (n.d.). 54 Mbps Wireless PCI Adapter. Wikimedia. Wikimedia Commons.

Degrees, 200. (n.d.). Bluetooth Connectivity. Pixabay. Pixabay.

Denver Public Library Community Technology Center. (2017, July 28). Home Networking 101 Lesson Plan. Denver, Colorado, United States of America.

Editor. (2020). Wireless Networking. Retrieved from Network Encyclopedia: https://networkencyclopedia.com/wireless-networking/

Estes, A. C. (2017, December 12). The history of wireless everything. Retrieved from Gizmodo: https://gizmodo.com/the-history-of-wireless-everything-1795227728

Fixr. (n.d.). Hardwired computer network cost. Retrieved from Fixr: https://www.fixr.com/costs/hardwired-computer-network

Frap. (2020, August 5). Frequency-hopping spread spectrum. Retrieved from Wikipedia: https://en.wikipedia.org/wiki/Frequency-hopping_spread_spectrum

Gechlik, K. (2010, March 18). A Simple Guide To Setting Up A Home Wireless Network. Retrieved from Make Use Of: https://www.makeuseof.com/tag/simple-guide-setting-home-wireless-network/

Goodman, P. (2020, April 27). Wireless network vs wired network: Advantages and disadvantages. Retrieved from TurboFuture: https://turbofuture.com/computers/Wireless-Network-vs-Wired-Network-Advantages-and-Disadvantages

Gookin, D. (n.d.). Hardware needed for a wireless network. Retrieved from Dummies: https://www.dummies.com/computers/pcs/hardware-needed-for-a-wireless-network/

Hassan, M. (n.d.). Wi-Fi Hotspot. Pxhere. Pxhere. Retrieved from https://pxhere.com/en/photo/1439809

Jongerius, J. (2020, October 22). Wi-Fi 4/5/6/6E (802.11 n/ac/ax). Retrieved from Duckware: https://www.duckware.com/tech/wifi-in-the-us.html#legacy

Kelly, Melissa. "Bloom's Taxonomy - Application Category." 25 January 2019. Thoughtco.com. https://www.thoughtco.com/blooms-taxonomy-application-category-8445. 16 August 2020.

LaBrie, G. (2017, November 21). 6 Benefits of wireless networking + wireless networking solutions. Retrieved from WEI Tech Exchange: https://blog.wei.com/6-benefits-of-wireless-networking-wireless-networking-solutions

Library of Congress. (1908). Guglielmo Marconi. Retrieved from Library of Congress: https://www.loc.gov/pictures/item/2004672066/

Lynn, S. (2013, November 12). Wireless Witch: How to place a wireless extender. Retrieved from PCMag: https://www.pcmag.com/news/wireless-witch-how-to-place-a-wireless-extender

Lynxman03. (n.d.). Běžná bezdrátová síť a ad hoc síť. Wikimedia. Wikimedia Commons.

macrovector. (n.d.). Smart City. Retrieved from Freepik: https://www.freepik.com/free-vector/city-wireless-internet-connection_1538535.htm#page=1&query=wireless%20city&position=3

macrovector. (n.d.). Wireless Network. Freepik. Freepik.

Mckinney, E. (n.d.). Disadvantages of wireless networks. Retrieved from Techwalla: https://www.techwalla.com/articles/disadvantages-of-wireless-networks

Mhandley, & Stannered. (n.d.). Wep-crypt-alt. Wikimedia. Wikimedia.

Miller, L. C. (n.d.). Wireless Security Protocols: WEP, WPA, and WPA2. Retrieved from Dummies: https://www.dummies.com/computers/computer-networking/wireless/wireless-security-protocols-wep-wpa-and-wpa2/

Mitchell, B. (2020, June 30). Everything you need to build a wireless network. Retrieved from Lifewire: https://www.lifewire.com/required-to-build-wireless-networks-816542

Mohamed, A. N. (2020, September 16). Orthogonal frequency-division multiplexing. Retrieved from Wikipedia: https://en.wikipedia.org/wiki/Orthogonal_frequency-division_multiplexing#Key_features

Myxomatosis57. (2020, May 1). Direct-sequence spread spectrum. Retrieved from Wikipedia: https://en.wikipedia.org/wiki/Direct-sequence_spread_spectrum

Net Spot. (2020). Wireless Security Protocols. Retrieved from Net Spot: https://www.netspotapp.com/wifi-encryption-and-security.html

Network Encylopedia. (n.d.). Wireless networking. Retrieved from Network Encylopedia: https://networkencyclopedia.com/wireless-networking/

Nokia Bell Labs. (n.d.). The first active orbiting communications satellite. Retrieved from Nokia Bell Labs: https://www.bell-labs.com/about/history-bell-labs/stories-changed-world/first-active-orbiting-communications-satellite/

pch.vector. (n.d.). Mix Raced Trainees Working on Computer in Public Library. Freepik. Freepik.

Pixaline. (n.d.). Smart Home. Pixabay. Pixabay.

RedPanda25. (2020, October 26). MIMO. Retrieved from Wikipedia: https://en.wikipedia.org/wiki/MIMO

Research and Markets. (2020, September 1). Global Wireless Technology Industry: Projected to Exhibit Explosive Growth Over 2020-2024 to Reach $5.52 Trillion in Revenues by 2024. Retrieved from PR Newswire: https://www.prnewswire.com/news-releases/global-wireless-technology-industry-projected-to-exhibit-explosive-growth-over-2020-2024-to-reach-5-52-trillion-in-revenues-by-2024--301122316.html

Riordan, M. (2020, March 26). Transistor. Retrieved from Encyclopaedia Britannica : https://www.britannica.com/technology/transistor#ref236464

Strickland, J. (2007, December 28). How ARPANET works. Retrieved from How Stuff Works: https://computer.howstuffworks.com/arpanet.htm

Tech Terms. (2018, March 14). NIC. Retrieved from Tech Terms: https://techterms.com/definition/nic

Techopedia. (2020). Cellular Network. Retrieved from Techopedia: https://www.techopedia.com/definition/24962/cellular-network

Techopedia. (2016, November 14). Wireless network. Retrieved from Techopedia: https://www.techopedia.com/definition/26186/wireless-network

The Editors of Encyclopaedia Britannica. (2015, September 24). Radiotelegraphy. Retrieved from Encyclopaedia Britannica : https://www.britannica.com/technology/radiotelegraphy

Schatz, G. (2016, February 8). The Complete List Of Wireless IoT Network Protocols. Retrieved from Link Labs: https://www.link-labs.com/blog/complete-list-iot-network-protocols

Schreyer Institute. "Online Assessment." 4 August 2020. sites.psu.edu/onlineassessment/. https://sites.psu.edu/onlineassessment/gather-evidence/#:~:text=%20Types%20of%20Online%20Assessments%20%201%20FORMATIVE,learning%20and%20their%20level%2Fskills.%20Lab%20simulations...%20More%20. 4 August 2020.

Snyder, J., & Thayer, R. (2004, October 4). Explaining TKIP. Retrieved from Network World: https://www.networkworld.com/article/2325772/explaining-tkip.html

TeachThought Staff. "What Is Bloom’s Taxonomy? A Definition For Teachers." 1 May 2020. https://www.teachthought.com/learning/what-is-blooms-taxonomy-a-definition-for-teachers/. Article. 6 August 2020.

Tech Target. (2007, September). Wireless protocols learning guide. Retrieved from Tech Target: https://searchnetworking.techtarget.com/tutorial/Wireless-protocols-learning-guide

Techopedia. (2020). Initialization Vector. Retrieved from Techopedia: https://www.techopedia.com/definition/26858/initialization-vector

Techopedia. (2016, November 14). Wireless Network. Retrieved from Techopedia: https://www.techopedia.com/definition/26186/wireless-network

The history of wireless technology- StoryMap. (n.d.). Retrieved from Exigent Networks: http://www.exigentnetworks.ie/the-history-of-wireless-technology-storymap/

ThinkSecureNet. (2018, February 19). 5 Uses for a wireless communication system. Retrieved from ThinkSecureNet: https://thinksecurenet.com/5-uses-wireless-communication-system/

Tom. (2020, February 8). Wireless access points - What they do & how they work. Retrieved from Smart Aerials: https://www.smartaerials.co.uk/blog/wireless-access-points-what-they-do-how-they-work

Tutorials Point. (2020). Advanced Encryption Standard. Retrieved from Tutorials Point: https://www.tutorialspoint.com/cryptography/advanced_encryption_standard.htm

Tutorials Point. (2020). Wi-Fi - IEEE Standards. Retrieved from Tutorials Point: https://www.tutorialspoint.com/wi-fi/wifi_ieee_standards.htm

What is an access point and how is it different from a range extender? (n.d.). Retrieved from Linksys: https://www.linksys.com/us/r/resource-center/what-is-a-wifi-access-point/

WorldWideSupply. (n.d.). The evolution of wireless network technology. Retrieved from Worldwide Supply: https://worldwidesupply.net/blog/blogevolution-wireless-network-technology/

Yambadwar, S. (2020, July 7). What is RC4 Encryption? Retrieved from Geeks For Geeks: https://www.geeksforgeeks.org/what-is-rc4-encryption/